CNN Accelerator Supporting Standard and Depthwise Convolution with ReLU and Max-Pooling

1.Introduction:

The proposed CNN Accelerator efficiently performs 3D convolution operations, supporting both standard convolution and depthwise convolution across multiple channels. It integrates pipelined MAC operations, ReLU activation, and max-pooling with on-chip BRAM-based memory management, enabling scalable processing of high-dimensional image data.

2. Key capabilities & design highlights:

- Parameterized Design: Configurable image size (n×n), kernel size (k, default 3), input channels (ch), filters (fi), memory address width (MA, default 18).

- 32-bit Signed Datapath: All pixel, weight, and accumulation values are 32-bit signed.

- Memory Interface: Dual-port BRAM (bram_dual_port1) for image/kernel reads and pooled output writes.

- Dedicated 3×3 MAC Core: mac9 computes patch-level multiply–accumulate with preloaded kernel weights.

- Index Generation & Line Buffering:

  - shift_index_generetor creates 3×3 window indices.

  - Line buffer stores rows for sliding windows to minimize external memory traffic.

- Nonlinear + Pooling: ReLU clamp → 2×2 Max Pool (max_pool4) with address/index generators.

- Multi-Channel/Filter Flow: FSM iterates over channels/filters, combines results, and sets completion flags.

- Synthesis Pragmas: (* ram_style="block", keep_hierarchy="yes" *) aid synthesis and optimization.

## 3.Control Flow (FSM Overview)

- s0 – Kernel Load: Stream kernel to karnel_matrix.

- s1 – Convolution: Load image pixels → line buffer → MAC9 → write conv outputs.

- s2 – Channel Update: Adjust addresses for next channel/filter.

- s3 – BRAM Align.

- s4 – ReLU + Pool: Apply ReLU, 2×2 max-pool, store outputs.

- s5/s6 – Filter Iteration: Repeat for all filters.

- s7 – Done/Idle.

## 4.Dataflow & Memory Mapping

- Latency Compensation: FSM accounts for MAC pipeline (~4 cycles).

- Dual-port BRAM: Port A for reads, Port B for pooled writes.

- Streaming Compute: Single mac9 reused for all patches (area-efficient).

- Memory Layout:

  - Image/kernel stored as contiguous words.

  - Kernel: k×k 32-bit values.

  - Pooled results start at maxpool_store_addre

For better understanding purpose image is taken from online so.

CNN Accelerator Pipeline Flow Chart

Hardware Utilization of 256×256 Matrix

Demonstration of Convolution Operation on a 256×256×3 Image Matrix with a 3×3×3 Kernel

The original image has been separated into three individual channels—Channel 1 (R), Channel 2 (G), and Channel 3 (B)—each with a resolution of 256 × 256 pixels.

import cv2

import numpy as np

# Input and output paths

img_path = r"D:\python file\standard_convolution\amp.webp"

out_file = r"D:\python file\standard_convolution\pixels_channelwise.txt"

# Step 1: Read the image

img = cv2.imread(img_path)

# Step 2: Resize to 256x256

img_resized = cv2.resize(img, (256, 256))

# Step 3: Split into channels (OpenCV loads as BGR)

B, G, R = cv2.split(img_resized)

# Step 4: Write pixel values to file (channel-wise)

with open(out_file, "w") as f:

# Write B channel

for row in B:

for val in row:

f.write(f"{val}\n")

# Write G channel

for row in G:

for val in row:

f.write(f"{val}\n")

# Write R channel

for row in R:

for val in row:

f.write(f"{val}\n")

print("Pixel file generated successfully at:", out_file)

by using this code I have generated  pixels_channelwise.txt this file .then convert into this hex file now this hex is uploaded to my CNN Accleretor(here depthwise and standard convolution possible_multichannel _multifilter possible).After simmuletion it generates  this  file .Again this file is uploaded into this python code.

import numpy as np

import cv2

# Input pixel file (single channel convolution output)

in_file = r"D:\memorey test file for cnn_accleretor\bram_dump6.txt"

# Image parameters

height = 254

width = 256

num_pixels = height * width  # 65024 pixels expected

# Step 1: Read pixel values

with open(in_file, "r") as f:

lines = f.readlines()

# Convert to integers

lines = [int(x.strip()) for x in lines]

# Step 2: Sanity check

if len(lines) != num_pixels:

raise ValueError(f"Pixel file has {len(lines)} values, but expected {num_pixels} ({height}x{width})")

# Step 3: Reshape into 2D matrix

img = np.array(lines, dtype=np.float32).reshape((height, width))

# Step 4: Normalize to 0–255 (for saving as image)

img_norm = cv2.normalize(img, None, 0, 255, cv2.NORM_MINMAX)

img_uint8 = img_norm.astype(np.uint8)

# Step 5: Save as PNG image

out_file = r"D:\python file\standard_convolution\multicha_convolution_output1.png"

cv2.imwrite(out_file, img_uint8)

print(f"Image generated successfully at: {out_file}")

This is expected                                                     Getting from CNN RLT

# Step 5: Define 3x3x3 kernel (example)

kernel = np.array([

[[0, -1, 0],

[-1, 5, -1],

[0, -1, 0]],   # for Red

[[0, -1, 0],

[-1, 5, -1],

[0, -1, 0]],   # for Green

[[0, -1, 0],

[-1, 5, -1],

[0, -1, 0]]    # for Blue

],

This 3*3*3  kernel I had used to my CNN_Accleretor.

Required Clock Cycles for 256×256×3   Convolution with 3×3×3 Kernel :

According to my design when system reach to in state 4  .Standard convolution operation is just completed  here we have performed 256*256*3    with  karnel 3*3*3  no. of  filter=1.

Required Clk=1966885/10 =1,96,688

According to my calculation=256*256*3=1,96,608  which is relatively quiet fast.

https://ieeexplore.ieee.org/document/10420491#:~:text=Optimizing%20CNN%20Computation%20Using%20RISC%2DV%20Custom%20Instruction%20Sets%20for%20Edge%20Platforms

According to this  IEEE  paper

  Achieves 7–8× higher performance compared to baseline.

  Provides greater hardware efficiency.

  Optimized datapath design for multi-channel/multi-filter operations.

  Efficient memory access and reuse reduce overhead.