Pixel-Level Multi-Channel Image Convolution verified RLT Using Python and OpenCV

The CNN Accelerator is a custom-designed hardware architecture that efficiently performs convolutional neural network (CNN) operations such as multi-channel convolution, ReLU activation, max pooling, and memory-optimized storage. Unlike generic processors or GPUs, this accelerator is highly specialized for deep learning workloads, focusing on throughput, latency reduction, and optimized memory utilization.

It leverages FPGA/ASIC-friendly design principles, such as dual-port BRAM-based storage, pipelined multiply-accumulate (MAC) units, and efficient address generation modules, to achieve parallelism at multiple levels (across pixels, channels, and filters). The design ensures scalability (supporting arbitrary image sizes n×n and kernel sizes k×k) and flexibility (configurable number of channels ch and filters fi).

By combining depthwise convolution flow, on-chip memory reuse, and streaming data handling, the accelerator achieves a balanced trade-off between area, speed, and power, making it suitable for real-time image processing and AI edge applications where conventional solutions fall short.

## Key Design Features of Your CNN Accelerator

### 1. Highly Parameterized Design

- n, k, ch, fi, MA, DEPTH all parameterized.

- Can scale image size (n*n), kernel size (k*k), number of channels (ch), and filters (fi) easily.

- Makes the same RTL reusable for multiple CNN layers (Conv, Pool, ReLU, Multi-Filter).

- Flexibility is a big advantage in hardware accelerators.

### 2. Block RAM (BRAM) Based Storage

- Using (* ram_style = "block" *) ensures efficient mapping to FPGA BRAM, saving LUTs/FFs.

- Dual-port BRAM (bram_dual_port1) allows parallel read/write:

  - Port A → Image/Kernel fetch

  - Port B → Convolution/Pooling write-back

  - This is more efficient than distributed RAM or registers.

### 3. Line Buffering for Sliding Window

- line[0:k-1][0:n-1] register array works as a line buffer.

- Avoids re-fetching pixels from external memory for every kernel window → huge memory bandwidth reduction.

- This mimics hardware-efficient image processing pipelines.

### 4. Shift Index Generators

- Modules like shift_index_generetor, pqindex_generetor, and pool_pq_index generate sliding window indices.

- They handle dynamic indexing in convolution and pooling without wasting cycles in nested loops.

- Saves control logic and improves parallelism.

### 5. Pipelined MAC (Multiply–Accumulate) Array

- mac9 module for 9 parallel multiplications + accumulation in one cycle.

- Data feeding + kernel weights + pipelined MAC ensures throughput of one convolution per cycle after pipeline fill.

- Critical for CNN acceleration.

### 6. Integrated Multi-Channel & Multi-Filter Handling

- chh_no and fi_no counters support:

  - Multi-channel convolution (RGB or more).

  - Multi-filter convolution (for multiple feature maps).

- Handles 3D convolution (depthwise + pointwise) inside one FSM.

- Saves control overhead, avoids external scheduling.

### 7. ReLU + MaxPooling in Hardware

- ReLU implemented as simple sign-check (if(dout_a[31]==1) relu=0 else relu=dout_a).

- max_pool4 computes max over 2x2 block.

- Pooling indices auto-generated, values written back to BRAM.

- Accelerator performs end-to-end CNN block (Conv → ReLU → Pool), not just convolution.

### 8. Memory Store Indexing

- mem_store_index automatically generates storage addresses for pooled results.

- Avoids address calculation overhead in main FSM.

- Makes write-back efficient.

### 9. FSM Controlled Operation

- Eight pipeline states (s0 → s7):

  - s0: Kernel loading

  - s1: Convolution (MAC)

  - s2: Channel adjust

  - s3: Address alignment

  - s4: ReLU + Pooling

  - s5: Multi-filter adjust

  - s6: Reset write address

  - s7: Completion

  - Clear separation of tasks → easier debugging, better hardware scheduling.

### 10. Scalability for FPGA/ASIC

- BRAM-based storage + FSM + parameterization → works on FPGA and scales to ASIC.

- Optimized for low memory bandwidth + high compute reuse, which is critical for CNN accelerators.

The original image has been separated into three individual channels—Channel 1 (R), Channel 2 (G), and Channel 3 (B)—each with a resolution of 256 × 256 pixels.

import cv2

import numpy as np

# Input and output paths

img_path = r"D:\python file\standard_convolution\amp.webp"

out_file = r"D:\python file\standard_convolution\pixels_channelwise.txt"

# Step 1: Read the image

img = cv2.imread(img_path)

# Step 2: Resize to 256x256

img_resized = cv2.resize(img, (256, 256))

# Step 3: Split into channels (OpenCV loads as BGR)

B, G, R = cv2.split(img_resized)

# Step 4: Write pixel values to file (channel-wise)

with open(out_file, "w") as f:

# Write B channel

for row in B:

for val in row:

f.write(f"{val}\n")

# Write G channel

for row in G:

for val in row:

f.write(f"{val}\n")

# Write R channel

for row in R:

for val in row:

f.write(f"{val}\n")

print("Pixel file generated successfully at:", out_file)

by using this code I have generated  pixels_channelwise.txt this file .then convert into this hex file now this hex is uploaded to my CNN Accleretor(here depthwise and standard convolution possible_multichannel _multifilter possible).After simmuletion it generates  this  file .Again this file is uploaded into this python code.

import numpy as np

import cv2

# Input pixel file (single channel convolution output)

in_file = r"D:\memorey test file for cnn_accleretor\bram_dump6.txt"

# Image parameters

height = 254

width = 256

num_pixels = height * width  # 65024 pixels expected

# Step 1: Read pixel values

with open(in_file, "r") as f:

lines = f.readlines()

# Convert to integers

lines = [int(x.strip()) for x in lines]

# Step 2: Sanity check

if len(lines) != num_pixels:

raise ValueError(f"Pixel file has {len(lines)} values, but expected {num_pixels} ({height}x{width})")

# Step 3: Reshape into 2D matrix

img = np.array(lines, dtype=np.float32).reshape((height, width))

# Step 4: Normalize to 0–255 (for saving as image)

img_norm = cv2.normalize(img, None, 0, 255, cv2.NORM_MINMAX)

img_uint8 = img_norm.astype(np.uint8)

# Step 5: Save as PNG image

out_file = r"D:\python file\standard_convolution\multicha_convolution_output1.png"

cv2.imwrite(out_file, img_uint8)

print(f"Image generated successfully at: {out_file}")

This is expected                                                     Getting from CNN RLT

# Step 5: Define 3x3x3 kernel (example)

kernel = np.array([

[[0, -1, 0],

[-1, 5, -1],

[0, -1, 0]],   # for Red

[[0, -1, 0],

[-1, 5, -1],

[0, -1, 0]],   # for Green

[[0, -1, 0],

[-1, 5, -1],

[0, -1, 0]]    # for Blue

],

This 3*3*3  kernel I had used to my CNN_Accleretor.

Hardware Utilization for    256*256 image matrix